HAZARD ASSESSMENT and PERSONAL PROTECTIVE EQUIPMENT SELECTION

| COMPANY NAME: XYZ Company DATE: March 2016 |  |  |  |
| --- | --- | --- | --- |
| PROTECTION AREA | LOCATION & ASSESSMENT OF HAZZARD | PERSONAL PROTECTIVE EQUIPMENT TO BE USED | TYPE EQUIPMENT |
| EYE PROTECTION Devices purchased after July 5, 1994 shall comply with ANSI Z87.1-1989 | Reconditioning Area – Chemical dust Parts Department – Impact, dust key maker Cutting torch – optical radiation Parts washers – chemical splash Brake washers – chemical splash Bench grinders – impact, dust Drill Press – impact Electric hand drills – impact Body Shop – impact, dust, grinding, chiseling, drilling, sanding Dynomometer - impact | Primary – Goggles Secondary – Face Shield Primary – Spectacles, goggles Secondary – Face Shield Spectacles or welding shield Primary – Goggles Secondary – Face Shield Primary – Goggles Secondary – Face Shield Spectacles, goggles, face shield Spectacles, goggles, face shield Spectacles, goggles, face shield Primary – spectacles, goggles Secondary – face shields Spectacles, goggles, face shields | Chemical splash proof goggles, face shield over primary eye protection. Spectacles with side protection goggles, face shields only over primary eye protection Filter lens, spectacles or goggles primary eye protection, welding helmets or face shields used primary eye protection Chemical splash proof goggles, face shield over goggles. Chemical splash proof goggles, face shield over goggles. Spectacles with side protection goggles, face shields only over primary eye protection. Spectacles with side protection goggles, face shields only over primary eye protection. Spectacles with side protection goggles, face shields only over primary eye protection . Spectacles with side protection goggles, face shields only over primary eye protection. Spectacles with side protection goggles, face shields only over primary eye protection. |

Areas of concern: Impact; Penetration; Compression (Roll-over);Chemical; Heat; Harmful Dust; Light (Optical) Radiation                Auto w/Body Shop

Name:___________________________________  Signature__________________________________     Date:_________

HAZARD ASSESSMENT and PERSONAL PROTECTIVE EQUIPMENT SELECTION

| COMPANY NAME: XYZ Company DATE: March 2016 |  |  |  |
| --- | --- | --- | --- |
| PROTECTION AREA | LOCATION & ASSESSMENT OF HAZZARD | PERSONAL PROTECTIVE EQUIPMENT TO BE USED | TYPE EQUIPMENT |
| HAND PROTECTION Devices purchased after July 5, 1994 shall comply with current requirements | Reconditioning shop - chemical Parts washer – chemical Brake washer – chemical Welding or cutting – heat Body Shop - chemical | Gloves Gloves Gloves Gloves Gloves | Impervious gloves* Neoprene, nitrile gloves Impervious gloves* Neoprene, nitrile gloves Impervious gloves* Neoprene, nitrile gloves Welding gloves Impervious gloves* Neoprene, nitrile gloves *per Material Safety Data Sheet |
| FOOT PROTECTION Devices purchased after July 5, 1994 shall comply with ANSI Z41-1991 | Service areas – impact Body Shop - impact | Safety shoes or boots Safety shoes or boots | Safety shoes or boots Safety shoes or boots |
| BODY PROTECTION | Battery Filling – chemical Body Shop – paint booth – chemical, dust | Apron Coverall suit | Impervious apron Impervious tyvek suit |
| HEARING PROTECTION | Service are & Body Shop – All excessive noise Dynomometer | Muffs or plugs Muffs or plugs | Muffs or plugs Muffs or plugs |

Areas of concern: Impact; Penetration; Compression (Roll-over);Chemical; Heat; Harmful Dust; Light (Optical) Radiation                Auto w/Body Shop

Name:___________________________________  Signature__________________________________     Date:_________

HAZARD ASSESSMENT and PERSONAL PROTECTIVE EQUIPMENT SELECTION

| COMPANY NAME: XYZ Company DATE: March 2016 |  |  |  |
| --- | --- | --- | --- |
| PROTECTION AREA | LOCATION & ASSESSMENT OF HAZZARD | PERSONAL PROTECTIVE EQUIPMENT TO BE USED | TYPE EQUIPMENT |
| FACE PROTECTION Devices purchased after July 5, 1994 shall comply with ANSI Z87-1989 | Cutting or welding – heat impact Parts and brake washers - chemical Battery filling – chemical, impact and all service Body Shop – impact, chemical, dust | Welding shields Face shields Chemical splash proof goggles, face shield Face shield | Welding shields used with primary eye protection Face shields used with primary eye protection Face shields used with primary eye protection Face shields used with primary eye protection Primary eye protection- Spectacles w/side protection or chemical splash proof goggles. |
| RESPIRATORY PROTECTION | Service area – brake, clutch and gasket service – dust Cutting & welding – dust, mist, and fumes Body Shop – dust, chemical, toxic dust, mist & fumes Paint Booth – dust, chemical, toxic dust, mist & fumes | Low pressure/Wet cleaning nega- tive pressure HEPA vacuum system Approved respirators Full face respirator w/approved cartridges or approved supplied air system* Respirators, supplied air hoods or masks | Brake wash, HEPA vacuum system. Approved dust, mist, fume Cartridges w/respirator. Full face respirator with cartridges or approved supplied air system* Approved supplied air systems or respirators for work being done.* Check MSHA/NIOSH ratings *per Material Safety Data Sheet |

Areas of concern: Impact; Penetration; Compression (Roll-over);Chemical; Heat; Harmful Dust; Light (Optical) Radiation                Auto w/Body Shop

Name:___________________________________  Signature__________________________________     Date:_________