XYZ Company

HAZARD ASSESSMENT/PPE MATRIX

| EQUIPMENT/TASK | POTENTIAL HAZARDS | RECOMMENDED PPE |
| --- | --- | --- |
| General Chemicals (Solvents, Petroleum Products, Cleaners, Degreasers, etc.) | Absorption Ingestion Inhalation | Splash goggles* Rubber gloves Dust Mask (if needed) |
| Battery Charging/Service | Battery Acid Contact | Rubber gloves Splash goggles / Face shield |
| Solvent Parts Washer | Chemical splash | Chemical neoprene gloves Splash goggles* |
| Vehicle Lift | Falling/crushing | Protective footwear (steel-toed or sturdy work boots) |
| Pneumatic Driving, Grinding, Cutting, etc., tools | Impact Excessive noise | Safety Glasses or goggles* Ear Plugs or muffs |
| Oxy-Acetylene Weld/Cut TIG/MIG Stick Welding Plasma Cutting | Optical radiation Burn/heat Fumes / Dust | Welding goggles or helmet Ventilation Welding gloves |
| Fuel Tank/Transmission Lift (portable) | Impact Crushing | Protective footwear (steel-toed or sturdy work boots) |
| Tire Changer | Explosion / Flying objects | Safety Glasses* |
| Tire Balancer | Falling objects Projectiles | Sturdy Work Boots Safety Glasses or goggles* |
| Brake Lathe | Impact Dust | Safety Glasses or goggles* Dust mask |
| Bench or Hand Grinders | Impact | Safety Glasses or goggles* |
| Electrical Hand Drill | Projectiles | Safety Glasses or goggles* |
| Spray Paint Operations (Spray Booth) | Inhalation Hazard Skin Absorption Chemical Splashes | Goggles Nitrile Gloves Negative Pressure Organic Vapor Respirator |

*NOTE:  Face shields may be worn as a supplement to safety glasses and goggles to provide added protection to the face, but may not be used alone as eye protection.  Face shields do not possess the impact and penetration properties required of eye protection.

Revised 9/2015